SRH UNIVERSITY

APPLIED MECHATRONICS

100007801_Aly_AP

Automated Pencil Production Line

Advanced Programming Final Project

Mohannad Aly
Student ID: 100007801
Course: Advanced Programming
Instructor: Esteban Pozo
Due date: June 20, 2026

A Python · InfluxDB · Grafana · Docker implementation

# Abstract

This project implements a software simulation of an automated pencil manufacturing line. The production sequence contains six stations: wooden-body feeding, graphite-core insertion, ferrule installation, eraser insertion, final quality inspection, and sorting. A Python state machine controls the sequence, creates simulated sensor values, records production statistics, and separates product-quality defects from equipment faults. The browser-based human-machine interface provides Start, Stop, Reset, Acknowledge, Emergency Stop, speed, and defect-probability controls. It also displays the current station, machine state, temperature, cycle time, first-pass yield, recent products, rejection reasons, and the event log. Production telemetry is written asynchronously to InfluxDB by using line protocol and is visualized through an automatically provisioned Grafana dashboard. Docker Compose packages the Python service, database, and dashboard as one reproducible system with persistent volumes. Automated unit and API tests verify the core controls, quality logic, fault handling, and web endpoints. The result demonstrates how programming, databases, visualization, version control, containerization, and artificial-intelligence-assisted development can be combined in a small manufacturing monitoring application.

Keywords: Python, manufacturing simulation, HMI, InfluxDB, Grafana, Docker, quality control

Submission package: The repository includes the source code, Docker configuration, Grafana provisioning, automated tests, GitHub Pages website, editable report, and final PDF.

# Contents

Abstract	2

Introduction	4

Task 1 - Product and Production-Line Concept	4

Task 2 - Program Development	5

Task 3 - Development Tools and Website	9

Task 4 - Conclusion and Further Development	10

References	11

Annex - AI Prompts, Outputs, and Corrections	12

Document structure: The main technical documentation occupies pages 4-10. References and the required AI-use annex follow separately.

# Introduction

Modern manufacturing software combines control logic, operator interaction, data collection, and visualization. This project applies those ideas to an educational production-line simulation. The objective is not to control real machinery; it is to demonstrate a clear software architecture in which a state machine produces repeatable process behavior, an HMI allows operator control, and time-series data supports monitoring and analysis.

The implementation uses only Python standard-library modules for the runtime web server, threading, queues, and simulation logic. Python provides threaded HTTP-server classes and concurrency tools suitable for educational network and I/O tasks, while its pseudo-random module supports repeatable simulated measurements when a seed is supplied (Python Software Foundation, 2026a, 2026b, 2026c). The project therefore remains easy to inspect and can be previewed even before the database containers are started.

| Task 1 - Product and Production-Line Concept | 10 points |
| --- | --- |

The selected product is a conventional wooden pencil with an eraser. It was chosen because the product has distinct mechanical components and measurable assembly characteristics. The simulated line contains more than the required four stages and models both assembly and inspection.

Wooden body feed: a pencil body is released from the feeder and checked for presence and damage.

Graphite core insertion: the core is inserted and checked for breakage and lateral misalignment.

Ferrule installation: the metal eraser holder is pressed onto the body and clamp force is evaluated.

Eraser insertion: the eraser is inserted into the ferrule and its presence and depth are checked.

Final quality inspection: overall length and surface quality are evaluated.

Sorting and discharge: accepted products and rejected products are routed to separate outputs.

Each pencil receives a product identifier and sensor-like measurements. A product that fails any quality checkpoint continues through the educational sequence but is marked for rejection at the sorting station. The rejection message identifies the exact reason, such as “Graphite core misaligned” or “Eraser missing.” Equipment faults are handled separately because a faulty machine should stop production, while a defective product should be counted and removed without stopping the line.

| Task 2 - Program Development | 60 points |
| --- | --- |

## 2.1 Software Architecture

The system is divided into the browser HMI, the Python application, InfluxDB, and Grafana. The Python application contains the production model, control API, and asynchronous database writer. Docker Compose starts all services and defines their network, environment variables, persistent storage, and startup dependencies. The use of health checks and dependency conditions follows the Compose approach for controlling service startup order (Docker, 2026).

Figure 1. System architecture and data flow (author’s own illustration).

The ProductionLine class owns the machine snapshot and protects it with a re-entrant lock. A dedicated production thread waits for the Start command and then executes the stage sequence. Stop pauses the current cycle, Reset clears counters when the line is not running, Acknowledge clears a fault, and Emergency Stop immediately changes the machine state. The HMI communicates through JSON endpoints such as /api/status, /api/start, /api/stop, and /api/settings.

## 2.2 Quality-Control Logic

A configurable final-product defect probability is distributed across five inspection points. Applying the full probability at every station would create a much higher final rejection rate, so the implementation calculates the equivalent per-stage probability as 1 - (1 - p)^(1/5). Once a defect is assigned, the corresponding measurement is changed to a realistic out-of-tolerance value and the reason is preserved until sorting.

| Station | Example measurement | Possible rejection reason |
| --- | --- | --- |
| Body feed | Body length / presence | Cracked body or missing body |
| Core insertion | Graphite offset | Misaligned or broken graphite |
| Ferrule press | Clamp force | Ferrule not seated / low force |
| Eraser insertion | Presence / depth | Missing or incorrect insertion depth |
| Final inspection | Length / surface score | Length or visual surface defect |

## 2.3 Human-Machine Interface

The frontend is a browser-based HMI implemented with HTML, CSS, and JavaScript. It includes the required Start, Stop, and Reset buttons and adds Acknowledge, Emergency Stop, speed control, defect-probability control, and a demonstration fault button. The page polls the Python API every 500 ms and updates the current stage, product number, counters, yield, motor temperature, cycle time, database connection, product table, and operator event log.

Figure 2. Browser-based production HMI showing the active stage and quality statistics (author’s own screenshot).

The interface uses color and text together so that state is not communicated by color alone. A fault changes the state indicator, displays a full-width alarm banner, writes a critical event, blocks a new Start command, and requires acknowledgement. The Emergency Stop uses the same controlled recovery path but retains a distinct EMERGENCY_STOP state. Product defects do not cause a machine fault; they are recorded in the recent-product table and increase the rejected counter.

## 2.4 Backend Robustness

The database client runs in a separate worker thread with a bounded queue. This prevents network delays from blocking the production cycle. Writes have a short timeout, one delayed retry, and periodic health checks. API request bodies are size-limited and validated, and dynamic text displayed by JavaScript is HTML-escaped. These measures are appropriate for a classroom simulation, although additional authentication and production-grade server software would be required in an industrial deployment.

## 2.5 InfluxDB Database and Grafana Dashboard

InfluxDB stores timestamped production telemetry in the pencil_production bucket. The application sends line-protocol records to the InfluxDB v2 HTTP write endpoint. Fields include temperature, total count, good count, defect count, cycle time, state code, stage index, defective-product flag, yield percentage, machine-state text, and defect reason. Tags identify the machine, state, stage, and event. InfluxData documents Docker-based setup and the HTTP API as supported methods for running and writing to InfluxDB v2 (InfluxData, 2026a, 2026b).

Grafana is automatically configured from YAML and JSON files mounted into its provisioning directories. The data source uses Flux with the organization, bucket, and token supplied through environment variables. The dashboard contains temperature and cycle-time trends, output statistics, first-pass yield, mapped machine states, and latest status information. Grafana supports provisioning an InfluxDB data source in configuration files, which makes the dashboard reproducible rather than dependent on manual setup (Grafana Labs, 2026).

Figure 3. Configured Grafana dashboard layout using representative project telemetry (author’s own visualization).

Docker services: pencil-hmi exposes port 8000, pencil-influxdb exposes port 8086, and pencil-grafana exposes port 3000. Named volumes preserve database and dashboard state between restarts.

## 2.6 Testing and Demonstration Results

The project contains automated tests based on Python’s unittest framework. The tests set a short cycle time and disable the external database so that logic can be verified quickly and deterministically. Eight tests were executed successfully during preparation of the submission package.

| Test area | Expected behavior | Result |
| --- | --- | --- |
| Start and production | At least one product completes; 0% defect setting produces only good products | Pass |
| Stop and reset | Stop changes state; reset clears counters when not running | Pass |
| Emergency stop | Restart is blocked until acknowledgement | Pass |
| Settings validation | Out-of-range speed and defect values are rejected | Pass |
| Injected machine fault | Line enters FAULT state with the specified message | Pass |
| Status API | JSON response contains state and temperature fields | Pass |
| Start API | POST request starts the production line | Pass |
| Static HMI | Root page serves the PencilLine interface | Pass |

A representative demonstration run produced good and defective pencils, displayed explicit rejection reasons, and maintained separate machine state and product result values. The production line remained responsive when InfluxDB was disabled because telemetry writes are decoupled from the main state-machine thread. When the Docker stack is running, database health changes to connected and the same fields become available to Grafana.

## 2.7 Program Files

| File or folder | Purpose |
| --- | --- |
| app.py | Starts the production model and threaded HTTP server. |
| production_line/engine.py | State machine, stages, defects, faults, counters, and telemetry. |
| production_line/influx_writer.py | Background InfluxDB health checks and line-protocol writes. |
| static/ | Browser HMI and JavaScript API client. |
| grafana/ | Provisioned data source and dashboard. |
| tests/ | Unit and API tests. |
| docker-compose.yml | Defines the three-service deployment and persistent volumes. |

| Task 3 - Development Tools and Website | 20 points |
| --- | --- |

Git and GitHub are used for version control, repository hosting, and project publishing. The repository separates source code, tests, infrastructure configuration, website files, and documentation so that changes can be reviewed logically. The included GitHub Actions workflow uploads the website folder and deploys it to GitHub Pages. GitHub Pages can publish from a branch or a custom Actions workflow; this project uses the workflow approach (GitHub, 2026).

Docker packages the Python application, InfluxDB, and Grafana into isolated services. The Dockerfile runs the application as a non-root user and includes an HMI health check. Docker Compose supplies credentials through environment variables, creates named volumes, waits for InfluxDB health before starting dependent services, and provides a one-command setup with docker compose up --build.

ChatGPT was used to support planning, code generation, review, test design, documentation, and visual consistency. OpenAI describes ChatGPT as an assistant that can support coding, writing, studying, and analysis tasks (OpenAI, 2026). All generated material was reviewed. Specific corrections included removing an unnecessary third-party framework, correcting the probability calculation, moving database writes to a background queue, adding HTML escaping and input validation, and clearly documenting security limitations. The relevant prompts, outputs, and corrections are included in the annex.

Project website: https://mohannad275.github.io/100007801_Aly_AP/

Repository: https://github.com/mohannad275/100007801_Aly_AP

Website content: The static site contains four explanatory paragraphs and two project images, meeting the assignment limit while providing direct links to the repository.

| Task 4 - Conclusion and Further Development | 10 points |
| --- | --- |

The project meets the requested functional scope: it simulates a manufacturing line with more than four stages, provides Python backend logic, identifies defective products and their reasons, implements the required HMI controls, writes production data to InfluxDB, and provisions a Grafana dashboard. The strongest aspect is the separation of concerns. Product assembly, machine state, operator controls, database communication, visualization, deployment, testing, and website publishing are contained in clear files and can be understood independently.

The HMI also gives a convincing demonstration of production behavior. Operators can observe the live sequence, control the line, alter simulation parameters, see accepted and rejected products, and deliberately test fault recovery. The background database writer means that a database problem does not freeze the process. Automated tests provide evidence that essential commands and API endpoints behave as intended.

The implementation has important weaknesses. It is a simulation rather than a real-time controller and therefore has no deterministic timing guarantee, programmable-logic-controller interface, physical safety circuit, or certified emergency-stop function. Sensor values and faults are generated statistically rather than received from hardware. The standard-library HTTP server is deliberately lightweight and is not recommended as a public production server; a deployed version would require a hardened application server, encrypted connections, user authentication, role-based authorization, network segmentation, secure secret handling, audit retention, backups, and maintenance procedures.

Further development could connect the software to an OPC UA server or PLC, add recipe management, maintenance work orders, alarm acknowledgement history, Overall Equipment Effectiveness calculations, database retention policies, Grafana alerts, and predictive-maintenance models. A real operational-technology solution must prioritize safety, availability, integrity, and controlled change. NIST guidance emphasizes that OT security must account for unique reliability, performance, and safety requirements (Stouffer et al., 2023). For this reason, the current application should be treated as an educational prototype and a basis for further engineering, not as a controller for physical machinery.

Overall assessment: The solution is complete for the academic brief, reproducible through Docker, transparent about AI use, and structured so that future PLC, SCADA, CMMS, and advanced quality modules can be added.

# References

Docker, Inc. (2026). Control startup and shutdown order in Compose. https://docs.docker.com/compose/how-tos/startup-order/

GitHub, Inc. (2026). Configuring a publishing source for your GitHub Pages site. https://docs.github.com/en/pages/getting-started-with-github-pages/configuring-a-publishing-source-for-your-github-pages-site

Grafana Labs. (2026). Configure the InfluxDB data source. https://grafana.com/docs/grafana/latest/datasources/influxdb/configure/

InfluxData. (2026a). InfluxDB HTTP API. https://docs.influxdata.com/influxdb/v2/api/

InfluxData. (2026b). Install InfluxDB using Docker Compose. https://docs.influxdata.com/influxdb/v2/install/use-docker-compose/

OpenAI. (2026). What is ChatGPT: FAQ. https://help.openai.com/en/articles/12677804

Python Software Foundation. (2026a). http.server - HTTP servers. https://docs.python.org/3/library/http.server.html

Python Software Foundation. (2026b). random - Generate pseudo-random numbers. https://docs.python.org/3/library/random.html

Python Software Foundation. (2026c). threading - Thread-based parallelism. https://docs.python.org/3/library/threading.html

Stouffer, K., Pease, M., Tang, C., Zimmerman, T., Pillitteri, V., Lightman, S., Hahn, A., Saravia, S., Sherule, A., & Thompson, M. (2023). Guide to operational technology (OT) security (NIST SP 800-82 Rev. 3). National Institute of Standards and Technology. https://doi.org/10.6028/NIST.SP.800-82r3

# Annex - AI Prompts, Outputs, and Corrections

The following prompts and outputs document the use of ChatGPT in accordance with the project brief. The output excerpts are condensed to the portions that directly influenced the submitted implementation.

## Prompt 1 - Overall Architecture

Prompt: Design a complete Python simulation of an automated pencil production line for an Advanced Programming project. It must include at least four assembly stages, explicit product-defect reasons, Start/Stop/Reset controls, an HMI, InfluxDB, Grafana, Docker, automated tests, and a clear separation between product defects and machine faults.

Relevant AI output:

Operator browser → Python HTTP API → ProductionLine state machine
                                  ↓
                             InfluxDB v2
                                  ↓
                               Grafana

States: IDLE, RUNNING, STOPPED, FAULT, EMERGENCY_STOP
Stages: body, graphite, ferrule, eraser, inspection, sorting

Correction and validation: The initial proposal used a third-party web framework. It was replaced by ThreadingHTTPServer to remove runtime dependencies. The InfluxDB writer was moved to a separate queue and thread so that a database delay cannot stop production.

## Prompt 2 - Quality-Control Rules

Prompt: Generate realistic quality-control measurements and defect reasons for body feeding, graphite insertion, ferrule pressing, eraser insertion, and final pencil inspection.

Relevant AI output: body length and presence, graphite offset and integrity, ferrule force, eraser depth and presence, final length, and visual surface score.

Correction and validation: Applying the configured defect probability at all five stations caused the final defect rate to be much too high. The probability was converted to a per-stage value using 1 - (1 - p)^(1/5), and automated testing was run with a zero-defect setting.

# AI Output Review and Benefits

## Prompt 3 - HMI, InfluxDB, and Grafana

Prompt: Create an industrial browser HMI with Start, Stop, Reset, Acknowledge, Emergency Stop, live process stages, production statistics, fault display, event log, recent product table, and settings. Add InfluxDB line-protocol fields and a provisioned Grafana dashboard.

Relevant AI output:

production_metrics,machine=pencil_line,state=RUNNING,stage=Ferrule_installation
  temperature=37.8,total_count=148i,good_count=137i,defect_count=11i,
  cycle_time=5.36,state_code=1i,yield_percent=92.6

Correction and validation: API input limits, numeric validation, HTML escaping, database timeouts, queue limits, health checks, and one retry were added. The HMI was run locally and the source was tested through its API endpoints.

## Prompt 4 - Testing and Documentation

Prompt: Write automated tests and a submission-ready report explaining the product, implementation, tools, weaknesses, AI use, and future improvements.

Relevant AI output: an eight-test plan, the Task 1-4 report structure, Docker demonstration steps, GitHub Pages content, and an AI correction log.

Correction and validation: All eight tests were executed successfully. The Word document was rendered to PDF, each page was visually inspected, and layout problems were corrected before delivery.

## Benefits of ChatGPT

ChatGPT accelerated brainstorming, scaffolding, test-case generation, documentation organization, and consistency checking across Python, JavaScript, Docker, InfluxDB, Grafana, the website, and the report. It was particularly useful for producing alternative designs and identifying edge cases. Human review remained essential because generated code can contain incorrect assumptions, excessive dependencies, unsafe defaults, probability mistakes, or deployment claims that have not been tested.

Final disclosure: ChatGPT was used as a development assistant. The submitted code and documentation were reviewed, tested, corrected, and assembled for this project. AI-generated material was not accepted without validation.